Land Corner Recordation Certificate

Authority: MCL 54.205

| Surveyor’s Name: |  |  | John A. Doe |  |  |  |  |  |  |  | Field Survey Date: | November 02, 2023 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| For corner(s) in: |  |  | Bear Creek Township, Emmet |  |  |  |  |  |  | County |  |  |
| Original Public Land Survey Corner |  |  |  |  |  |  |  |  | Corner Code |  |  |  |
|  | S | 22 |  | T | 34N | R | 06W |  | G-09 |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
| Property Controlling Corner |  |  |  |  |  |  |  |  | Corner Code |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
| Property Corner |  |  |  |  |  |  |  |  | Corner Code |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
| Protracted Public Land Survey Corner |  |  |  |  |  |  |  |  | Corner Code |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |

Part A:  Corner History

Land Corner Recordation Certificate Liber 5 Page 163, Emmet County Records

Emmet County Remon monument 2” X 36" with magnetic base and brass cap in Monument Box

- (1)Spike in West face Power Pole Stub		N20°W	89.64’

- (2)Pipe in Concrete					East	32.95’

- (3)Tag in East face 7” Pine				N55°E 	55.43’

- (4)Tag in South face Telephone Pole stub		N20°W	89.19’

MDOT Plan Set

US-31, CS 24011, JN 110605, 2014, Station 367+05.62 Offset 1420.78 RT

Standard Emmet County Remon Monument in Monument Box

- (1)Spike in West face cutoff Power Pole		N23°W	89.54’

- (2)Pipe in Concrete Monument			East	33.00’

- (3)Nail and Tag in East face 7” Pine			N57°E	55.38’

- (4)Tag in South face cutoff Power Pole		N30°W 	89.40’

N=746,760.31.75	E=19,523,414.51  MCS Central Zone NAD83(2011) Epoch Date: 2010

Part B:  Surveyor’s Report on Perpetuation, Restoration, Reestablishment, or Monumentation of Corner

Pre-Construction Witnesses as measured 04/03/22

Found Standard Emmet County Remon Monument in Monument Box

- (1)Found Spike in West face cutoff Power Pole	N23°W	89.55’

- (2)Found Pipe in Concrete Monument		N90°E	33.11’

- (3)Found Nail and Tag in East face 7” Pine		N57°E	55.35’

- (4)Found Tag in South face cutoff Power Pole	N32°W 	89.25’

- (5)Set Nail in Hub 			 		S10°E 	167.75’

- (6) Set Mag Nail and Tag in Asphalt Parking Lot	N90°E	129.03’

N=746,760.31.70	E=19,523,414.55 MCS Central Zone NAD83(2011) Epoch Date: 2010

Post-Construction as measured 11/02/2023

Set 5/8” X 36” rebar with Cap 39084 and new Monument Box

- (1)Found Spike in West face cutoff Power Pole	N23°W	89.54’

- (2)Found Pipe in Concrete Monument		N90°E	33.64’

- (3)Found Nail and Tag in East face 7” Pine		Removed during Construction

- (4)Tag in South face cutoff Power Pole		Removed during Construction

- (5)Found Nail in Hub				S10°E 	167.76’

- (6)Found Mag Nail and Tag in Asphalt Parking Lot	N90°E	129.02’ Removed after Construction

Surveyors Report:

Prior to construction the corner was verified with witnesses and coordinates by comparing to Remon LCRC and MDOT plans.  The corner was removed or destroyed during construction activities on US-31.  The corner was replaced using RTK CORS observations and verified and adjusted using the Pre-Construction witnesses. Witness (2) Pipe in Concrete Monument was hit by construction equipment and leaning to the East after construction.

Part C:  Field Evidence of Perpetuation or Monumentation of Corner

5/8” X 36” rebar with Cap 39084  in monument box

- (1)Found Spike in West face cutoff Power Pole	N23°W	89.55’

- (2)Found Pipe in Concrete Monument		N90°E	33.64’

- Set Spike and Tag in 8” cedar Fence Post		N90°W	43.55’

- Set Mag Nail and Tag in 24” White Pine		S45°W	66.66’

|  |  |  |  | Certifying Surveyor’s Seal/Stamp |
| --- | --- | --- | --- | --- |
| John A. Doe |  |  | Date |  |
| Professional Surveyor’s License No.: | 0123456789 |  |  |  |
| Prepared By: Jason Doe, 1234 US-31, Traverse City, MI, 49685 |  |  |  |  |