Land Corner Recordation Certificate

Authority: MCL 54.205

| Surveyor’s Name: |  |  | John A Doe |  |  |  |  |  |  |  | Field Survey Date: | October 17, 2023 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| For corner(s) in: |  |  | Bear Creek Township, Emmet |  |  |  |  |  |  | County |  |  |
| Original Public Land Survey Corner |  |  |  |  |  |  |  |  | Corner Code |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
| Property Controlling Corner |  |  |  |  |  |  |  |  | Corner Code |  |  |  |
|  | S | 27 |  | T | 35N | R | 05W |  | PC500 |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
| Property Corner |  |  |  |  |  |  |  |  | Corner Code |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
| Protracted Public Land Survey Corner |  |  |  |  |  |  |  |  | Corner Code |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |
|  | S |  |  | T |  | R |  |  |  |  |  |  |

Part A:  Corner History

MDOT P.I. alignment control for Highway M-119 in the Southeast 1/4 of Section 27, T35N, R05W, Bear Creek Township, Emmet County, Michigan

MDOT Plan set, M-119, CS 24051, JN 27842A & 37818A, PC STA 867+53.09

- 1/2” re-rod, buried 0.1’

- NW cor conc sign base				S30°E	72.50’

- P Pole						S15°W	73.86’

- P Pole						N47°W	60.74’

- (1)12” Maple					N55°E	75.67’

Part B:  Surveyor’s Report on Perpetuation, Restoration, Reestablishment, or Monumentation of Corner

Pre-Construction Witnesses as measured 03/15/2023

Found 5/8” rebar in driveway approach

- (1)SW face of 18” Maple				N57°E	75.3’

N=760738.35’	E=19541027.23’ MCS Central Zone NAD83(2011) Epoch Date: 2010

Straight-line Witnesses as measured 03/15/2023

- Set mag nail in asphalt parking lot			S34°E	75.67’

- Set 5/8” rebar with MDOT Traverse Point cap	S34°E	98.61’

- Set 5/8” rebar with MDOT Traverse Point cap	S34°E	172.65’

- Set 5/8” rebar with MDOT Traverse Point cap	S34°E	243.57’

Post-Construction Straight-line Witnesses as measured 10/17/2023

- Mag nail in asphalt parking lot			Removed during construction

- Found 5/8” rebar with MDOT Traverse Point cap	S34°E	98.60’

- Found 5/8” rebar with MDOT Traverse Point cap	S34°E	172.67’

- Found 5/8” rebar with MDOT Traverse Point cap	S34°E	243.56’

Surveyors Report:

Prior to construction the corner was verified with witnesses, coordinates, and by comparing found monumentation to record information in MDOT plans.  The corner was removed or destroyed during construction activities on M-119.  The corner was reestablished using RTK CORS observations and verified and adjusted using the Pre-Construction straight-line witnesses.

Part C:  Field Evidence of Perpetuation or Monumentation of Corner

Set 5/8” X 36” rebar with Cap 39084 in monument box

- (1)Set nail & tag in SE face of 18” Maple		N58°E	75.94’

- Set nail & tag in NW face PP			S20°W	76.97’

- Set nail & tag in SE face PP				N44°W	57.66’

- NW corner building foundation			S33°W	121.55’

|  |  |  |  | Certifying Surveyor’s Seal/Stamp |
| --- | --- | --- | --- | --- |
| John A. Doe |  |  | Date |  |
| Professional Surveyor’s License No.: | 0123456789 |  |  |  |
| Prepared By: Jason Doe, 1234 US-31, Traverse City, MI, 49685 |  |  |  |  |